Supplementary  Table 1. Measurands and methods of included CBC parameters.

| Analytes | Methods |
| --- | --- |
| Erythrocytes(RBC), x 1012/L | Counted (Hydrovynamic vocusing (DC Detection) / Empedans) |
| Hemoglobin (Hgb), g/L | Measured (SLS-Hemoglobin Method) |
| Hematocrit (HTC), % | Calculated, HTC=(1k) × Pi A ×100 A: Total volume of the blood sample k: Constant |
| MCH, pg | Calculated, MCH= Hgb (gdL)RBC (x106µL)x10 |
| MCHC, g/dL | Calculated, MCHC=Hgb gdLHTC % x10 |
| MCV, fL | Calculated, MCV= HTC %RBC (x106µL)x10 |
| RDW-SD, fL | Calculated (X2-X1) With the peak height assumed to be 100%, the distribution width at the 20% frequency level is RDW-SD. |
| RBC-He, pg | Scattergram ® RBC-He is a measure of the haemoglobin content of mature red blood cells |
| Reticulocyte, x109/L | Counted (Flow cytometry method using semiconductor laser) |
| RET-He, pg | Scattergram ® RET-He is a measure of the haemoglobin content in reticulocytes |
| Leukocytes, x109/L | Counted (Flow cytometry method using semiconductor laser) |
| Lymphocytes, x109/L | Counted, (Flow cytometry method using semiconductor laser) |
| Monocytes, x109/L | Counted (Flow cytometry method using semiconductor laser) |
| Neutrophils, x109/L | Counted (Flow cytometry method using semiconductor laser) |
| Eosinophils, x109/L | Counted (Flow cytometry method using semiconductor laser) |
| Basophils, x109/L | Counted (Flow cytometry method using semiconductor laser) |
| PLT-I, x109/L | Counted (Hydroynamic vocusing (DC Detection)/ Empedans) |
| PDW – SD, fL | Calculated, (Y2-Y1) With the peak height assumed to be 100%, the istribution width at the 20% frequency level is PDW-SD. |
| Plateletcrit, % | Calculated, Plateletcrit=(1k) × Pi B ×100 B: Total volume of PLT in sample k: Constant |
| MPV, fL | Calculated, MPV= PCT %PLT (x103µL)x10000 |
| P-LCR, % | Calculated, The P-LCR is the ratio of large platelets from the 12 fL discriminator or larger. It is calculated as a ratio comparing the number of particles between the fixed discriminator and UD, to the number of particles between LD and UD. |