Appendix

Various model equations used for estimating membrane transport parameters

Kedem-Katchalsky model [47]

Solvent flux, Jw:	Jw=Lp*ΔP-σΔπs,          Δπs=RT(Cm-Cp)	………….. (21)

Solute flux, Js:	Js=ωΔπs+Cs(1-σ)Jw				………….. (22)

where

ΔP is hydraulic pressure difference and Δπs is the osmotic pressure difference.

Cf, Cm, Cp are solute concentrations at feed side, membrane, and the permeate side

Cs is the logarithmic mean of Cm and Cp.

Model constants Lp, σ, ω are specific hydraulic permeability, reflection coefficient, and local solute permeability.

Spiegler-Kedem model [48]

Solute rejection, R:	R=σ(1-F)(1-σF)						….... (23a)

F=exp1-σJwP=exp1-σJwω RT						….... (23b)

where P represents solute permeability.

Jitsuhara andKimura model for charged membranes [41]

Reflection coefficient, σ for a negatively charged membrane [47]

σ=Δp-ΔΠiΔΠsJw=0=1-X'2ΔCsZII-ZI-2t10-1ln2t10-1+ZII2t10-1+ZI		…... (24)

where        Z=1+2CsX'2

Reflection coefficient, σ based on Jitsuhara and Kimura model is given by:

σ=1+ϕX'ΔCsZII-ZI-2t1-1ln2t1-1+ZII2t1-1+ZI			………….. (25)

where Z= 1+B,         B=2CsϕX'2,     X'=Xϕw,     	………….. (26)

X is the charge density of the membrane,

ϕ is osmotic coefficient, and ϕX’ is effective charge density, t is the transport number [49],

and the superscripts I and II represent the higher and lower pressure sides.

The other transport parameter - solute permeability P is determined from:

P=RTϕX'ϕwk2F2ΔXCsICsII1+B-1)CsdCsCsICsII1+B-1)1+B+1). 1λ10+1λ20dCs					………….. (27)

where, ϕw is the water content of the membrane, k2 is tortuosity factor,

ϕw/k2ΔX is the parameter used instead of numerous individual parameters ϕ, X’, k2, ΔX [50].

Combined film theory Spiegler-Kedem (CFSK) model:

Observed solute rejection, Robs

Ln1-RobsRobs=Ln1-RR+Jwk						………….. (28)

where k the coefficient of mass transfer.

Observed solute rejection, Robs [51] is given by:

Robs=11-σ1-exp-1-σPJwexpJwk+1					…………..(29)

After simplication Robs [33] can represented as:

Robs1-Robs=a11-exp⁡-Jwa2expJwk			…………..(30)

a1= σ1-σ								…………..(31a)

a2=1-σP								…………..(31b)

with maximum rejection observed at:

Jw,min=kln1+PePe							…………..(32)

where Pe is peclet number given by:

Pe=1-σkP								…………..(33)

Jw,min is the point where the rejection will be maximum and is equal to the mass transfer coefficient k.