PITAC checklist

Patient label:

PITAC procedure no.:

Date:

| Procedure | Yes | No | Comments |
| --- | --- | --- | --- |
| - WHO Safe surgery checklist documented? |  |  |  |
| - Correct chemotherapy type, dose and patient ID? |  |  |  |
| - Spill Kit available? |  |  |  |
| - All medical staff wear glasses or other kind of eye protection? |  |  |  |
| - Surgeons and scrub nurses wear double layered gloves and protective barrier garments? |  |  |  |
| - Floor beneath injector covered by non-permeable blanket? |  |  |  |
| - Container for hazardous waste in place? |  |  |  |
| - Exsufflation kit with two micro particle filters inserted between the 5 mm trocar and the closed main ventilation system? |  |  |  |
| - 5 mm trocar closed? |  |  |  |
| - Thermoflator activated with heated CO2? |  |  |  |
| - Intrathoracic pressure of 12 mmHg? |  |  |  |
| - CO2 flow rate < 0,2 liters/minute? |  |  |  |
| - Subcutaneous emphysema present? |  |  |  |
| - PLM score and malignant pleural effusion volume documented in REDCap? |  |  |  |
| - Pleural lavage performed? |  |  |  |
| - Pleural biopsies taken? |  |  |  |
| - CE-certified nebulizer flushed with saline? |  |  |  |
| - CE-certified nebulizer inserted and fixated through the 12 mm trocar pointing away from lung tissue? |  |  |  |
| - High pressure line tightly connected to the syringe and CE-certified nebulizer? |  |  |  |
| - High pressure line covered by plastic bag? |  |  |  |
| - Injector settings with a maximum of 300 PSI and flow-rate of 0.5-1.8 ml/sec? |  |  |  |
| - Remote control to the injector present? |  |  |  |
| - Monitors visual through the window? |  |  |  |
| - Anesthesiology - ready for treatment? |  |  |  |